Secure Access Entry System

system_with_1_serial_port folder files

These files take input parameters and also shows debug outputs on same serial port

system_with_2_serial_ports_folder files

Takes input from Serial port2 and shows output on serial port2

- Upload ESP32_ENTRY_WITHOUT_BUSY_STATE.ino code to the esp32 and open the serial monitor

- Copy the command from line 10

“erase” and enter into the serial monitor

- All datas will get erased and the system restarts

Next copy the line 11

{"ssid":"stijo"}

{"ssid":"yourSSID"}

- Replace it with your ssid and enter into serial monitor

- Output will be new Wifi ssid set ,next silimlary copy 12th line and set password

- It will show connecting wifi,then Connected to wifi network

- Make sure you put the correct ssid and password

Possible error situtaion

- If you get any connectivity issue even after entering correct ssid and password, that will be solved on next a device reset

- Now set the remaining parameters from line 13

Similary set gymid,deviceid

{"devid":"0001"}                       //set the device id

{"gymid":"1"}                          //set the gymid

{"serid":"0000000010110011"}          //set the service id

{"clockset":"16;20;23;05;07;21"}   //manual clock set if needed

{"clockset":"HH;MM;SS;DD;MM;YY"}

inclock                         //set the internet clock to rtc

clock                           //printout current rtc clock  datas

load                            //printout the ssid,password,deviceid,gymid,servicecode stored in eeprom

Load command output

- Press reset button of the Esp32 ,so that device reset

ssid  set

password  set

deviceid  set

gymid  set

serviceid  set

date set

- if any of these paramaters are not set try reconfiguring using its particular commands

- “net ok”  this keeps on outputting onscreen which means internet and wifi connectivity is ok

- If internet connectivity is lost “no internet” is printed out

- If wifi accesspoint is not there or any connectivity issue

- Copy the first line , set the current date

{"gid":"1","mid":"1","sc":"0000000010110011","at":"I","rid":"1234567876","dt":"12092021","st":"00:00","et":"23:00"}

{"gid":"1","mid":"1","sc":"0000000010110011","at":"I","rid":"1234567876","dt":"DDMMYYYY","st":"00:00","et":"23:00"}

- Then enter it into serial monitor

- If your paramters on flash matches the incoming json parameters the output will be this

- And based on internet connectivity it will send it to the server or will write it into the flash file

Eg:data uploaded to server

Eg: Data appended to file due bad internet connectivity

Eg:Automatic update data from file to server when internet connectivity comes back